CHAPTER 5

# STRUCTURAL ANALYSIS

Once the preliminary design is completed, the structure undergoes analysis for various load cases and combinations. The results from this analysis are used to refine the section designs, incorporating more precise data on load effects. These sections are iteratively adjusted to comply with code requirements and professional standards.

## 5.1 METHOD OF ANALYSIS

ETABS is a structural analysis and design software that can be used to analyze a building structure. The analysis methods available in ETABS include:

### 5.1.1 Linear Static Analysis:

ETABS is a software tool for structural analysis and design, specifically tailored for analyzing building structures. It offers various analysis methods to evaluate the behavior and performance of buildings.

### 5.1.2 Multi-Step Static Analysis:

This type of analysis is used to analyze structures that are subjected to loads that are applied in a series of steps. For example, a structure that is subjected to wind loads that increase gradually over time would be analyzed using multi-step static analysis.

### 5.1.3 Dynamic Analysis:

Structures that are subject to dynamic loads, such as impact and earthquake loads, are analyzed using this kind of analysis. Linear or nonlinear techniques can be used for dynamic analysis. The kind of structure being examined and the kinds of stresses it is subjected to determine the kind of analysis employed by ETABS. For instance, linear static analysis can be used to examine a straightforward beam that is subjected to gravitational stresses. Nonlinear static analysis, however, can be required to examine a complex structure that is subjected to dynamic loads.

The following steps are involved in performing an analysis in ETABS:

- Create the structural model.

- Define the loads and boundary conditions.

- Select the analysis method.

- Run the analysis.

- Analyze the results.

□ Create the structural model:

This initial step involves defining the geometry of the structure within ETABS. It includes specifying the layout of beams, columns, slabs, walls, and other structural elements. The model also incorporates information about material properties such as concrete, steel, etc., and how these elements are connected or interact within the structure.

□ Define the loads and boundary conditions:

In this step, the engineer specifies the various loads that act on the structure. These can include dead loads (self-weight of the structure), live loads (occupant loads), wind loads, earthquake loads, temperature effects, etc. Boundary conditions are also set to define how the structure interacts with its supports or surroundings, such as fixed supports, pinned supports, and constraints due to adjacent structures.

□ Select the analysis method:

ETABS offers different analysis methods depending on the nature of the structure and the types of loads applied. Common methods include static analysis (linear or nonlinear), dynamic analysis (response spectrum analysis, time history analysis), and nonlinear analysis (pushover analysis, nonlinear time history analysis). The choice of method depends on factors like structural complexity, loading conditions, and design requirements.

□ Run the analysis:

This step involves executing the analysis within ETABS. The software solves the equations derived from the structural model and the specified loads and boundary conditions. During this process E-tabs computes the internal forces, Stresses, deformations and other relevant responses of the structures under the applied loads.

Analyze the results:

After completing the analysis, engineers review and interpret the results generated by ETABS. This process assesses whether the structure meets safety requirements, evaluates its performance under the applied loads, and verifies compliance with design criteria and building codes. This step may involve examining graphical outputs, numerical data, and detailed reports provided by the software to make informed decisions about the structure's design and behavior.

## 5.2 CALCULATION OF BASE SHEAR

### 5.2.1 Base Shear Coefficient

Table 1GFT

| Horizontal Base shear Coefficient Calculation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| As per NBC 105:2020 [MRF] |  |  |  |  |  |
| Height of Building (H) = | 19.200m | (Height of the building from foundation or from top of a rigid basement.) |  |  |  |
| Seismic Zone Factor (Z) = | 0.3 | Ref (Cl 4.1.4) |  |  |  |  |
| Importance Factor (I) = | 1.25 | Ref (Cl 4.1.5) |  |  |  |  |
| Soil Type = | B | Soil type B |  |  |  |  |
| Structure type = | Moment resisting concrete frame |  |  |  |  |  |
| (Kt)= | 0.075 | Ref (Cl 5.1.2) |  |  |  |  |
| Approximate Fundamental Period of Vibration: |  |  |  |  |  |  |
| (T1) = KtH0.75= | 0.688Sec | Ref (Cl 5.1.2) |  |  |  |  |
| Amplification of approximate period: |  |  |  |  |  |  |
| T1=1.25*T1= | 0.868 | Ref (Cl 5.1.3) |  |  |  |  |
| From Rayleighs |  |  |  |  |  |  |
| T1= | 0.680Sec | Ref. no. Cl. 5.1 |  |  |  |  |
| Method of Analysis= | Response Spectrum Analysis |  |  |  |  |  |
| Lower period of flat part of spectrum |  |  |  |  |  |  |
| Ta = | 0.1 | Ref (Cl 4.1.2) |  |  |  |  |
| Lower period of flat part of spectrum |  |  |  |  |  |  |
| Tc = | 0.7 | Ref (Cl 4.1.2) |  |  |  |  |
| Peak spectral acceleration normalized by PGA |  |  |  |  |  |  |
| α = | 2.5 | Ref (Cl 4.1.2) |  |  |  |  |
| Coefficient that controls the descending branch of the spectrum |  |  |  |  |  |  |
| K = | 1.8 | Ref (Cl 4.1.2) |  |  |  |  |
| Spectral shape factor |  |  |  |  |  |  |
| Ch(T) = | 2.5 | Ref (Cl 4.1.2, Eq. 4.1(2)) |  |  |  |  |
| Elastic site spectra for horizontal loading |  |  |  |  |  |  |
| C(T)=Ch(T) Z I= | 0.9375 | Ref (Cl 4.1.1, Eq. 4.1(1)) |  |  |  |  |
| Elastic site spectra for Serviceability Limit State (SLS) |  |  |  |  |  |  |
| Cs(T)= 0.2* C(T) = | 0.1875 | Ref (Cl 4.2, Eq. 4.2(1)) |  |  |  |  |
| Ductility Factor |  |  |  |  |  |  |
| Ductility factor for Ultimate Limit State (ULS) |  |  |  |  |  |  |
| Rμ = | 4 | Ref (Cl 5.3.1, Table 5-2) |  |  |  |  |
| Overstrength Factor |  |  |  |  |  |  |
| Overstrength factor for ULS |  |  |  |  |  |  |
| Ωu = | 1.5 | Ref (Cl 5.4.1, Table 5-2) |  |  |  |  |
| Overstrength factor SLS |  |  |  |  |  |  |
| Ωs = | 1.25 | Ref (Cl 5.4.2, Table 5-2) |  |  |  |  |
| Horizontal base shear coeffieicent for ULS, |  |  |  |  |  |  |
| Cd(T1)=C(T1)/(Rμ*Ωu)= | 0.156 | Ref (Cl 6.1.1, Eq. 6.1(1)) |  |  |  |  |
| Horizontal base shear coeffieicent for SLS, |  |  |  |  |  |  |
| Cd(T1)=Cs(T1)/Ωs = | 0.150 | Ref (Cl 6.1.2, Eq. 6.1(2)) |  |  |  |  |
| Exponent related to the structural period (K) |  |  |  |  |  |  |
| K = | 1.090 | Ref (Cl 6.3) |  |  |  |  |
| Accidental Eccentricity |  |  |  |  |  |  |
| e = | 0.100 | Ref (Cl 5.7) |  |  |  |  |

## 5.3. BASE REACTION SCALING:

When the design base shear (VR) obtained by combining the modal base shear forces is less than the base shear (V) calculated using Equivalent Static Method, the member forces, story shear forces & base reactions obtained from the MRS method shall be multiplied by VR/V.

Where, V = Base Shear determined from Equivalent Static Method VR = Base Shear determined from Modal Combination

| Table 2: Base Reaction TABLE: Base Reactions. Output Case; Case Type; Step Type; Step Number; FX; FY; FZ; MZ. kN; kN; kN; kN-m. Modal; LinModEigen; Mode; 1; 384.4772; -3106.29; 0; -60040.3. Modal; LinModEigen; Mode; 2; 4016.867; 569.841; 0; -87598.9. Modal; LinModEigen; Mode; 3; 489.504; -2092.92; 0; 28933.42. Modal; LinModEigen; Mode; 4; 319.555; -9496.47; 0; -183090. Modal; LinModEigen; Mode; 5; 13668.38; 8042.563; 0; -377578. Modal; LinModEigen; Mode; 6; -14836.7; 7352.433; 0; 265416.1. Modal; LinModEigen; Mode; 7; -461.667; -11852.7; 0; -257593. Modal; LinModEigen; Mode; 8; -937.271; -16323.1; 0; 72575.03. Modal; LinModEigen; Mode; 9; 1875.935; -12998.2; 0; -228683. Modal; LinModEigen; Mode; 10; -3344.43; 14513.78; 0; 654322.8. Modal; LinModEigen; Mode; 11; 55412.89; -809.679; 0; -1212186. Modal; LinModEigen; Mode; 12; 3329.46; 28704.11; 0; -112223. Dead; LinStatic; 0; 0; 36776.92; 0. FF; LinStatic; 0; 0; 6066.723; 0. Wall Load; LinStatic; 0; 0; 21030.5; 0. LIVE; LinStatic; 0; 0; 14607.87; 0. EQX ULS; LinStatic; -9878.81; 0; 0; 263407.3. EQX SLS; LinStatic; -9483.66; 0; 0; 252871. EQY ULS; LinStatic; 0; -9878.81; 0; -110263. EQY SLS; LinStatic; 0; -9483.66; 0; -105853. Roof Live; LinStatic; 0; 0; 0; 0. SPECX; LinRespSpec; Max; 9881.534; 777.8213; 0; 259376.6. SPECY; LinRespSpec; Max; 954.4783; 9874.791; 0; 157404.3. Water Tank; LinStatic; 0; 0; 0; 0. 1.2DL+1.5LL; Combination; 0; 0; 98560.77; 0. DL+0.3LL+EQX (ULS); Combination; -9878.81; 0; 68256.5; 263407.3. DL+0.3LL-EQX (ULS); Combination; 9878.808; 0; 68256.5; -263407. DL+0.3LL+EQY (ULS); Combination; 0; -9878.81; 68256.5; -110263. DL+0.3LL-EQY (ULS); Combination; 0; 9878.808; 68256.5; 110263.4. DL+0.3LL+SPECX; Combination; Max; 9881.534; 777.8213; 68256.5; 259376.6. DL+0.3LL+SPECX; Combination; Min; -9881.53; -777.821; 68256.5; -259377. DL+0.3LL+SPECY; Combination; Max; 954.4783; 9874.791; 68256.5; 157404.3. DL+0.3LL+SPECY; Combination; Min; -954.478; -9874.79; 68256.5; -157404. DL+LL; Combination; 0; 0; 78482.01; 0. |  |
| --- | --- |

|  |  | Mode | 1 | 384.4772 | -3106.289 | 0 |  |  | -60040.2587 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mode | 2 | 4016.8672 | 569.841 | 0 |  |  | -87598.9115 | 0 | 0 | 0 |
|  |  | Mode | 3 | 489.504 | -2092.9235 | 0 |  |  | 28933.4243 | 0 | 0 | 0 |
| ## 5.4 CENTRE OF MASS AND CENTRE OF RIGIDITY CHECK |  | Mode | 4 | 319.555 | -9496.4658 | 0 |  |  | -183090.0128 | 0 | 0 | 0 |
| Table 3: Centre of Mass and Rigidity TABLE: Center Of Mass And Rigidity. Story; Diaphragm; XCCM; YCCM; XCR; YCR. m; m; m; m; ex; ey; ex%; ey %. Story1; D1; 0; 0; 0; 0; 0; 0. Story2; D2; 8.9825; 22.4829; 7.6807; 22.2679; 1.302; 0.215; 2.232222; 0.126. Story3; D3; 10.153; 22.4843; 7.2153; 22.259; 2.938; 0.2253; 9.32056; 1.20. Story4; D4; 9.0147; 22.483; 7.0145; 22.2264; 2; 0.2566; 6.11222; 0.04. Story5; D5; 9.015; 22.4827; 7.0814; 22.1967; 1.934; 0.286; 5.74222; 0.36. Story6; D6; 9.0244; 22.5041; 7.2098; 22.2885; 1.815; 0.2156; 5.08111; 0.5. Story7; D7; 3.4631; 22.5; 2.6748; 22.4755; 0.788; 0.0245; 4.379444; 0.012. |  | Mode | 5 | 13668.3824 | 8042.5634 | 0 |  |  | -377578.2465 | 0 | 0 | 0 |
|  |  | Mode | 6 | -14836.6996 | 7352.4334 | 0 |  |  | 265416.1141 | 0 | 0 | 0 |
|  |  | Mode | 7 | -461.6674 | -11852.6778 | 0 |  |  | -257592.6184 | 0 | 0 | 0 |

## 5.5 IRREGULARITIES IN BUILDING

### 5.5.1 Mass Irregularity

According to NBC 105:2020, if the seismic weight of any level is greater than 150% of the seismic weight of the floor below, the structure is deemed to have center of mass abnormalities. Buildings that exhibit mass irregularity are more susceptible to environmental forces such as wind loads, earthquakes, and other disasters, since it can result in structural instability. When catastrophic occurrences occur, irregularities might lead to high-stress zones where the building may fail or collapse.

Figure 2: Mass irregularity

Table 4 Mass irregularities check

| Story | Mass X or y | Mass <50% of Above or below story |
| --- | --- | --- |
| Story6 | 657439 | OK |
| Story5 | 1151424 | OK |
| Story4 | 1171160 | OK |
| Story3 | 1273943 | OK |
| Story2 | 1177060 | OK |
| Story1 | 1284695 | OK |
| Base | 175761 | - |

### 5.5.2 STIFFNESS/RIGIDITY IRREGULARITY

As per NBC 105:2020, Center Stiffness irregularity is present in the structure when the lateral stiffness of a story is less than that of the story above. This irregularity is also known as a soft story. Soft stories are more prone to collapse or deform under lateral loads. In an earthquake, a soft story can fail catastrophically, leading to the collapse of the entire building.

Table 5 Stiffness/Rigidity Irregularity

| Story | Stiff x kN/m |  | (Ki <0.7ki+1) | (Ki<(0.8K(i+n)/n) |
| --- | --- | --- | --- | --- |
| Story6 | 257177.2 | OK |  | OK |
| Story5 | 529368 | OK |  | OK |
| Story4 | 676777 | OK |  | OK |
| Story3 | 833772.8 | OK |  | OK |
| Story2 | 938060.5 | OK |  | OK |
| Story1 | 5838719 | OK |  | OK |
| Base | 175761 | OK |  | OK |

Table 6 Story Stiffness

| TABLE: Story Stiffness |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Story | Output Case | Stiff Y | (Ki<0.7ki+1) | (Ki<(0.8K(i+n)/n) |  |  |
|  |  | kN/m |  |  |  |  |
| Story6 | EQY ULS | 280859.218 | OK |  | OK |  |
| Story5 | EQY ULS | 509435.099 | OK |  | OK |  |
| Story4 | EQY ULS | 589408.859 | OK |  | OK |  |
| Story3 | EQY ULS | 644406.418 | OK |  | OK |  |
| Story2 | EQY ULS | 1100383.272 | OK |  | OK |  |
| Story1 | EQY ULS | 7524573.709 | OK |  | OK |  |

## 5.6 STOREY DRIFT CHECK

Storey Drift is the ratio of the inter storey displacement to the storey height under consideration. As per the recommendations of NBC 105: 2020 5.6.3, The ratio of the inter-story deflection to the corresponding story height shall not exceed: 0.025 at ultimate limit state 0.006 at serviceability limit state The deflections shall be obtained by using the effective stiffness properties of the components. It also safeguards the non-structural components of the structure.

Table 7 Storey Drift Check

| Drift Criteria (NBC 105:2020) |  |  |  |
| --- | --- | --- | --- |
| Design Drift from Etabs in ULS | = | 0.005036 | 0.005536 |
| Allowable drift ULS=.025/Ru | = | 0.00625 | 0.00625 |
|  |  | OK | OK |
| Design Drift from Etabs in SLS | = | 0.004835 | 0.005314 |
| Allowable Drift SLS | = | 0.006 | 0.006 |
|  |  | OK | OK |

Figure 3 Story Drift EQx ULS

Figure 4Story Drift EQy ULS

Figure 5 Story Drift EQx SLS

Figure 6 Story Drift EQY SLS

## 5.7 DISPLACEMENT CHECK

The displacement at the top storey shall not exceed:

0.025 × H at ultimate limit state.

0.006 × H at serviceability limit state.

|  | X(mm) | Y(mm) |
| --- | --- | --- |
| Design Displacement from Etabs in ULS | 59.683454 | 58.298037 |
| Allowable Ratio ULS | 0.006 | 0.006 |
| Allowable Displacement ULS=0.025H/Ru | 140 | 140 |
| Design Displacement from Etabs in SLS | 57.296116 | 55.96625 |
| Allowable ratio SLS | 0.025 | 0.025 |
| Allowable Displacement SLS=0.006H | 134.4 | 134.4 |

Figure 7 Displacement EQX ULS

Figure 8 Displacement EQyULS

Figure 9 Displacement EQx SLS

Figure 10 Displacement EQy SLS

## 5.8 MODAL MASS PARTICIPATING RATIO

The modal mass participating ratio is the effective mass in a structure that is used for the analysis of the earthquake. The number of modes considered for the analysis should be such that the total seismic mass of the structure is at least 90% of the total mass. The first three modes, considered along the principal directions, suggest that the contribution of any further mode to the total seismic mass does not exceed 10%, as per NBC 105: 2020. Part of the check moves the effective masses in the two principal horizontal directions.

Table 8 Modal Participating Mass Ratio

| TABLE: Modal Participating Mass Ratios |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case | Mode | UX | UY | UZ | SumUX | SumUY | SumRZ |
| Modal | 1 | 0.0104 | 0.4779 | 0 | 0.0104 | 0.4779 | 0.1668 |
| Modal | 2 | 0.6653 | 0.0159 | 0 | 0.6757 | 0.4937 | 0.1672 |
| Modal | 3 | 0.0065 | 0.1561 | 0 | 0.6822 | 0.6498 | 0.6566 |
| Modal | 4 | 0.0001 | 0.0593 | 0 | 0.6822 | 0.7091 | 0.6868 |
| Modal | 5 | 0.1098 | 0.007 | 0 | 0.792 | 0.7162 | 0.6968 |
| Modal | 6 | 0.0222 | 0.0325 | 0 | 0.8143 | 0.7486 | 0.7459 |
| Modal | 7 | 0.00001759 | 0.0144 | 0 | 0.8143 | 0.763 | 0.7595 |
| Modal | 8 | 0.0001 | 0.0143 | 0 | 0.8143 | 0.7774 | 0.77 |
| Modal | 9 | 0.0001 | 0.0046 | 0 | 0.8144 | 0.782 | 0.7705 |
| Modal | 10 | 0.0002 | 0.0034 | 0 | 0.8146 | 0.7854 | 0.7927 |
| Modal | 11 | 0.057 | 0.000004008 | 0 | 0.8717 | 0.7854 | 0.7928 |
| Modal | 12 | 0.0001 | 0.0138 | 0 | 0.8718 | 0.7993 | 0.798 |
| Modal | 13 | 0.000004201 | 0.0047 | 0 | 0.8718 | 0.804 | 0.8019 |
| Modal | 14 | 0.00003744 | 0.00004122 | 0 | 0.8718 | 0.8041 | 0.8019 |
| Modal | 15 | 0.00003747 | 0.0024 | 0 | 0.8718 | 0.8064 | 0.8093 |
| Modal | 16 | 0.0584 | 0.00000153 | 0 | 0.9303 | 0.8064 | 0.8093 |
| Modal | 17 | 0.0408 | 0 | 0 | 0.9711 | 0.8064 | 0.8099 |
| Modal | 18 | 0.0033 | 0 | 0 | 0.9744 | 0.8064 | 0.812 |
| Modal | 19 | 0.0006 | 0 | 0 | 0.975 | 0.8064 | 0.8995 |
| Modal | 20 | 0 | 0 | 0 | 0.975 | 0.8064 | 0.8995 |
| Modal | 21 | 0.00000641 | 0 | 0 | 0.975 | 0.8064 | 0.8995 |

## 5.9 MODAL PERIOD AND FREQUENCIES

In the modal response spectrum method, The modal period (time for one vibration cycle) and its reciprocal-the frequency. These parameters guide the earthquake resistant design. The base shear forces are determined for each mode, considering the first few significant modes.

Table 9Modal Periods And Frequencies

| TABLE: Modal Periods And Frequencies |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case | Mode | Period | Frequency | UX | UY | SumUX | SumUY |
|  |  | sec | cyc/sec |  |  |  |  |
| Modal | 1 | 0.869 | 1.151 | 0.008 | 0.5208 | 0.008 | 0.5208 |
| Modal | 2 | 0.815 | 1.227 | 0.6748 | 0.0136 | 0.6828 | 0.5344 |
| Modal | 3 | 0.754 | 1.327 | 0.0073 | 0.1337 | 0.6901 | 0.6681 |
| Modal | 4 | 0.294 | 3.397 | 0.0001 | 0.0641 | 0.6902 | 0.7322 |
| Modal | 5 | 0.243 | 4.122 | 0.0613 | 0.0212 | 0.7514 | 0.7534 |
| Modal | 6 | 0.242 | 4.14 | 0.0709 | 0.0174 | 0.8223 | 0.7708 |
| Modal | 7 | 0.183 | 5.453 | 2.28E-05 | 0.015 | 0.8224 | 0.7859 |
| Modal | 8 | 0.155 | 6.446 | 4.81E-05 | 0.0146 | 0.8224 | 0.8005 |
| Modal | 9 | 0.133 | 7.503 | 0.0001 | 0.005 | 0.8225 | 0.8055 |
| Modal | 10 | 0.121 | 8.237 | 0.0002 | 0.0043 | 0.8228 | 0.8098 |
| Modal | 11 | 0.117 | 8.522 | 0.0551 | 0.00001176 | 0.8779 | 0.8099 |
| Modal | 12 | 0.114 | 8.805 | 0.0002 | 0.013 | 0.878 | 0.8228 |
| Modal | 13 | 0.092 | 10.922 | 4.1E-06 | 0.0052 | 0.878 | 0.828 |
| Modal | 14 | 0.087 | 11.471 | 3.01E-05 | 0.00004764 | 0.8781 | 0.8281 |
| Modal | 15 | 0.086 | 11.638 | 3.35E-05 | 0.0022 | 0.8781 | 0.8303 |
| Modal | 16 | 0.076 | 13.111 | 0.0539 | 0.000008958 | 0.932 | 0.8303 |
| Modal | 17 | 0.062 | 16.048 | 0.0396 | 0.000004217 | 0.9715 | 0.8303 |
| Modal | 18 | 0.055 | 18.211 | 0.0035 | 0.00001102 | 0.9751 | 0.8303 |
| Modal | 19 | 0.053 | 18.862 | 0.0006 | 0.0016 | 0.9756 | 0.8319 |
| Modal | 20 | 0.044 | 22.799 | 1.42E-05 | 0.1193 | 0.9756 | 0.9512 |
| Modal | 21 | 0.041 | 24.619 | 0 | 0.000000513 | 0.9756 | 0.9512 |

Maximum allowable value for ∑UX and ∑UY crossing 90% should be less than 33Hertz.

## 5.10 BEAM-COLUMN CAPACITY RATIO

The beam-column capacity ratio (BCC Ratio) compares the moment capacity of a beam framing into a column at that joint. Essentially, it checks whether the column can handle the forces transmitted by the beams without failing.

Figure 11Beam-column capacity ratio

The value for BCC Ratio should be less than 1.2 and vice versa for CBB ratio.

## 5.11 MAT FOUNDATION FROM ETABS

### 5.11.1 Bearing Capacity Check

Bearing capacity refers to the soil's ability to support the loads applied to it from structures. The allowable bearing capacity was 200kN/M² while we got 120 KN/m²."

Figure 12 Soil pressure check

### 5.11.2 Settlement Check

Settlement is defined as the vertical movement of a foundation due to applied loads and soil behavior. The maximum settlement obtained was 12.8 cm while allowable was 65cm.

Figure 13 Settlement Check

### 5.11.3 Punching Shear Check

Punching Shear is a failure where concentrated load near columns causes a shear force that exceeds the footing capacity to resist it. Our punching shear is below 1.00.

Figure 14 Punching Shear Check